# Homework 2, ET4605, course 2025-2026 “LNA design.” NEW

For a low-noise 2.xx GHz wireless receiver, we want to design an LNA. Since we aim for a commercial wireless application, the DC power consumption of the LNA must be minimized while assuming a supply voltage of 3V. The lower the DC power consumption, the better; creativity is rewarded at this point. However, please note the specifications below, which your design must satisfy.

Note that for this project, both:

the device characterization and LNA design are required

The specifications for the LNA are as follows.

Operating frequency = 2.GHz + (0.last two digits student number)GHz

e.g., if your student ID is 1234567, the operating frequency is: 2.67GHz

F < 1.2 dB

S11 < -15 dB

S22 < -15 dB

S21 > 18 dB

K> 1 for all frequencies

For the implementation of these circuit blocks, we have received the Gummel-Poon parameters of a bipolar “reference” transistor from our foundry, with a relative active emitter area of 1. We will use scaled versions of this device to implement the circuit blocks.

Important: ADS templates for an easy start with this project are available on Brightspace.

You can also start from scratch using the Gummel Poon parameters below.

#### Step 1) Device characterization (20% score of homework set 3)

Questions / Deliverables:

- Determine the fT and Fmin of the reference transistor using a VCE of 3 V as a function of the collector current (use log scale), and explain the choice of your simulation frequency. What is the best biasing point for low noise?

- Is this also the point for achieving high transistor gain?

- Plot the K-factor as a function of frequency for the optimum bias point, 
Is the device unconditionally stable?

Tips

- Set all the variables roughly correct and customize the sweep range

- To plot fT, insert an equation on your presentation page and use the built-in ADS function  (stoh(S)) to convert to hybrid parameters, the way you present your simulation result should look rather similar to what we have used in Chapter 2)

#### Step 2) An inductively degenerated LNA stage (60% score of homework set 3)

The goal of this step is to achieve simultaneous noise and impedance matching conditions for the device technology described above. For this purpose, we recommend using the circuit topology shown below. Try to find the optimal combination of component values, device scale, and collector current that yields a design meeting the LNA specifications above.

Figure 2. An inductively degenerated LNA stage.

Background theory for the LNA matching problem)

To get going with this problem, consider the simplified AC schematic of the bipolar device with inductive degradation in its emitter, below. To simplify the problem, we assume that we have no input conductance (gbe) and no feedback capacitance (Cbc). Furthermore, for the convenience of our calculations, we assume that the output is AC short-circuited.

Zin

Yopt

L

Cbe

Gm.Vbe

inoise

Iout

Active device

Figure 3. Strongly simplified schematic of an active device to be matched simultaneously for noise and impedance.

To achieve simultaneous noise and impedance at its input terminal, we need to address the following steps/questions

TIPs

- Select first the optimum current density for the given device technology (is this current density providing you with sufficient gain?), try to keep this current density constant when scaling your device. How do you do this?

- Try to obtain conjugate matching conditions at the base of your transistor for the input reflection coefficient or the optimum noise match. To achieve this goal, modify Le and the transistor area strategically. No equation for the noise match is required since this is rather complicated.

- Finally, try to match using the proper value of Lb

Remark: perfect matching conditions are sometimes difficult to achieve; be practical at this point.

#### Deliverables

- Values of Lb, Le, scaling factor, and collector current

- Simulation evidence that you have indeed achieved the simultaneous noise and impedance matching conditions at the design frequency

- Make the device unconditionally stable.

- Match the output to 50 Ohm and explain briefly (max ten lines) how you did it and why you used this specific solution.

#### Questions

- What is the approximating equation for the input match 
when assuming Cbc = 0?

- What is the resulting gain (s21)? Is it equal to MaxGain?

- Does your stabilization network degrade the noise figure? If so, how much?

- After adding the output match, do you have to tune the input matching again?
If so, why is that?

- Why does the emitter inductance has much more impact on the input impedance than the noise matching condition?

- How does the F of the LNA compare to the Fmin of the device?

- Why is inductive degeneration better for noise than resistive degeneration?

#### Step 3) “low power LNA” (20% score of homework)

After you have successfully completed Part 3, you have found an optimum DC current for your LNA, which, combined with your DC supply voltage, yields a certain DC power consumption for your LNA.

- Next, give a clear strategy on how to lower the power consumption of your LNA (avoid just tweaking your previous solution). A schematic design change is needed to reach this goal. Modify your previous LNA schematic and show the drop in DC power consumption, while maintaining your noise level (F).

- Briefly outline the pros and cons of your solution.

## About this homework

ADS templates for steps 1, and 2 are available on Brightspace

Creativity and unique solutions (if they make sense) will be positively rewarded!

The turn-in date will be Jan 9!

Success with this problem!

Merry Christmas and Happy New Year!

Leo