# Homework 3, EE4605, 2025-2026,

Problem I (100 points), “A low distortion Differential BJT Output Stage”

To get you started, we provide the circuit topology (see below) and allow you to use a close-to-ideal bipolar device. The output capacitance of the transistors is included as the lumped capacitances of 9 pF each.

- Please use the template that contains the proper settings of the harmonic balance simulation.

Your design frequency is 1.9 GHz

Figure 1: Idealized highly linear differential LNA.

Figure 2: Active device parameters, all default values except the current gain.

To motivate independent work,

- The current gain Beta forward of the bipolar transistor is BF=1y0

- Your supply voltage VDD = 2 + 0.5 x  z .

Where y, and z are the last two numbers of your student ID, e.g., if your student ID is 1234567 ( y=6, z=7), then the current gain Beta forward of the bipolar transistor is BF=170 and the supply Voltage is 4.8V.

Understanding the transformer

We assume an ideal center-tapped transformer. For this purpose, we utilize ideal inductors with ideal coupling (K=1) between all windings and employ a high inductance (e.g., >1mH) for Lprim and Lsec.

- Determine how Lprim and Lsec must be chosen (using simulations) for a balun function offering an unbalanced-to-differential impedance transformation of 1:1.

To check your conclusions, use the suggested schematic below.

- What can you say about the impedance the secondary winding offers to common-mode signals resulting from the differential stage? (How would you simulate it to verify your statement?)

DC, AC, S-parameters, and matching

As stated, the loading your differential amplifier needs to see, provided by the transformer, is a 12.5 ohm differential.

- Determine the value of the inductor (Linshin) to resonate out the output capacitance at 1.9GHz

- Hand-calculate the output current Imax (see slide Class-B) to reach compression.

Bias your amplifier in class AB/class B by choosing Vbe such that it is at ~20% of the calculated maximum current (Imax) when reaching P1dB

For this quiescent condition

- Calculate your input impedance, and match your input using (a) l/4 transmission line(s), (give the calculation and show s11 in small signal conditions)

- Match the output balun transformer to 50 ohm so that the differential impedance offered to the transistor pair is still 12.5 ohm at the center frequency.

- Check the RF bandwidth of your design, provide s21 and s11

- What is the stability factor K?

Large-signal single-tone operation

Simulate the P1dB (or the power where your amplifier starts to compress. It does not need to be precisely P1dB) of your amplifier.

- Plot your load line; does it make the full swing, does it have a lot of hysteresis? 
(If it exhibits a lot of hysteresis, please verify that your inductor value was set correctly to resonate out the output capacitance).

- Does P1dB agree with your hand calculation, using the supply voltage related to your student id.?

- Plot the drain efficiency vs the output voltage on a linear plot.

- What is the drain efficiency when your amplifier enters compression? Does it agree with your expectations? Explain why / why not?

- Is it a straight line vs. Vout? Explain why / why not?

- Plot the AM-AM and AM-PM transfer.

  - What is your AM-AM/ gain level? Is it flat vs. power? Explain why/why not.

Linearity simulations and optimization

Rcenter is initially set to zero.

Apply a two-tone simulation power sweep, using a delta f of 10 MHz.

- Plot the IM3 products in dBm vs. Pout

- What is the OIP3 if we use extrapolation of the IM3 products?

Plot also the AM-AM and IM3 for the two-tone signal in dBc vs fundamental output power (as on slide xx)

Rcenter is no longer set to zero.

- Determine the optimum Rcenter and Vbe to obtain a flat AM-AM and low IM3

- Does it agree with the theory of slide?

- Explain why / why not.

- What is the IP2 of your design? (Explain why it is so high/so low).

Make now the area of the T2 1% larger than that of T1;

- What happens with the IP2 and IP3? Explain why the impact of this change is much larger for one of them.

# Problem 2 (100 points), “An asymmetric Doherty Amplifier in class-B operation”

We will implement an Asymmetric Doherty Power Amplifier, operating in Class-B.

- To keep this homework problem manageable in terms of time and difficulty, we will use an ideal device and an ADS template.

- To motivate independent work, the maximum output power level and back-off level of your DPA are dependent on your student ID. Namely;

- The maximum power level of the main amplifier Doherty amplifier using class B operation should be equal to: (10 + x) W

- The high efficiency back-off point of your Doherty amplifier should be located at –(7+0,5 . y) dB power back-off

In which x and y are the last two digits of your student ID, so if your student ID is 1234567, then the maximum output power should be equal to 16 W and the high-efficiency power back-off should be located at -10.5dB.

- The operating frequency of the DPA = 1 GHz

- The supply voltage of the Doherty amplifier is 10V for both the peak and main device.

Design tips

- Check the lecture notes!

- Calculate the design parameters, Pout@back, , Imain_max, Imain_peak. Note that the input voltage amplitude is normalized to 1V, so calculate and set gm to obtain the desired current levels at peak output power.

- Calculate the value of RL and the value of the characteristic impedance Ro of the Doherty impedance inverter.

- Do not forget to match the Doherty output impedance (RL) in the end to a 50ohm load, by dimensioning the /4 impedance transformer at the output.

- Now check the operation of the Doherty

- Use the ADS template!

- Check first if the behavior of Imain and Ipeak is correct (yes -> next step), if not, first check the equation for the control voltages and the values of gm

- Next, check if the behavior of voltages is correct; if not, check the Ro and RL values.

The Design

- The initial ADS template provides the harmonic shorts and Doherty power combiner; it also includes an extra l/4 line to achieve impedance matching to 50 ohms.

  - Calculate using lecture notes, the maximum currents for the main and peak branches.

  - Following the design tips, provide the deliverables for the class-B operation, showing the correct output power and efficiency back-off point

  - Can you provide some suggestions on how to improve the efficiency of your Doherty amplifier even more?

Deliverables,

Schematic of your amplifier with all component values filled in

Simulation evidence that the amplifier works correctly, namely.

- Imain and Ipeak versus input voltage

- Vmain and Vpeak versus input voltage

- Fundamental loading conditions of both main and peaking devices versus input voltage

- Efficiency versus output power

- Time domain waveforms for the current through the main and peak devices and their drain voltages, next plot the loadline for main device at different output power levels.

Success with this problem!

Leo